Plan Commission Staff Report						             July 20, 2021

Department of Planning and Building

Town of McCordsville

Project:	Summerton, Section 1 Development Plan & Secondary Plat

Petitioner: 		Lennar Homes

Request:	Petitioner is seeking approval of the Development Plan and Secondary Plat for Section 1 of Summerton

Staff Review:	Section 1 consists of 48 lots on approximately 26 acres.

Subject Property Zoning

The property is zoned Summerton PUD.

Surrounding Land-uses & Zoning

The subject Real Estate is currently undeveloped.  The surrounding land uses are as follows:

- North: Future phases of Summerton

- East: Across CR 500W is mostly undeveloped land zoned County R-1 and County IL

- South: Across CR 900N is agricultural property zoned County R-1

- West: Future phases of Summerton

Infrastructure

All lots will be accessed through the internal public streets.  In accordance with the preliminary plan, this section will feature one entrance on CR 500W and two street stubs to future phases.  The project will install a perimeter sidewalk along both CR 500W and CR 900N.  Stormwater detention is provided through a master drainage plan for Summerton.  Section 1 will include one stormwater detention pond.  Sanitary sewer service will be provided by the Town, and water service will be provided by Citizen’s Energy Group.

Development Standards

The PUD includes its own Development Standards.  The larger PUD includes two lot types; however, all of Section is included in Area B.

Area B

Max Number of Lots:			116

Min. Lot Area:				8,450 SF

Min. Lot Width @ B.L.:			65’

FY Setback:				25’

SY Setback:				7.5’

RY Setback: 				15’

Min. Livable Floor Area:		1,500 SF/1,800 SF

Min. Ground Floor Living Area:		900 SF

Max. Primary Structure Height:		35’

Max Lot Coverage:			50%

Perimeter Landscaping:  Perimeter landscaping featuring 7 trees, 6 shrubs, and 3-5’ tall mounding is required along CR 500W and CR 900N.  In addition, the adjoinder parcels also receive a required landscape treatment of evergreen trees planted 40’ on-center (O.C.)  The petitioner’s plans meet this requirement.

Lot Landscaping: Each unit has must include 3 trees and 12 shrubs, with additional requirements for corner lots, and certain garage protrusions.  In addition, sod is required for front yards.  This item is reviewed for after each home is constructed.

Architecture: All homes must comply with the Summerton PUD.  All homes will be reviewed on permit-by-permit basis when submitted for building permit review.

Street Lighting: Street lighting, consistent with the Town’s Zoning Ordinance is required.  The petitioner’s is currently updating the plan set to account for the Town’s revised street light standard.

Pedestrian Accessibility:  The PUD requires internal sidewalks on both sides of all internal streets and a perimeter sidewalk along CR 900N & CR 500W.  The petitioner’s plans meet this requirement.

Open Space:  Open Space is required in accordance with the Preliminary Plan.  The petitioner’s plans are in compliance.

Entrance Median: The PUD requires a median at the entrance.  The petitioner’s plans meet this requirement.

Entrance Signage: The PUD requires primary signage monumentation at the entrance.  The petitioner’s plans show the primary monumentation.

Technical Advisory Committee (TAC)

The project went to TAC on June 3, 2021 and received a number of comments.  Staff is currently wrapping a few final comments with the petitioner.  Those comments are related to street lighting and “no parking” signage.   Staff will see that these remaining issues are completed prior to the meeting.  The petitioner will continue to work with staff on setting addresses, street names, and stormwater concerns.

Staff Comments:	Staff is supportive of this petition and following resolution of the items listed above will recommend approval.  Following discussion, the Commission can motion to provide (a) approval, (b) continuance, or (c) denial.