Plan Commission Staff Report						            June 21, 2022

Department of Planning and Building

Town of McCordsville

Project:	Jacobi Farms, Section 1 Development Plan & Secondary Plat

Petitioner: 		Pulte Group

Request:	Petitioner is seeking approval of the Development Plan and Secondary Plat for Section 1 of the Jacobi Farms residential subdivision

Staff Review:	Section 1 features eighty-two (82) lots on approximately 31 acres.  This Section features two product types which are detailed below.

Subject Property Zoning

The property is zoned Jacobi Legacy Farms PUD.

Surrounding Land-uses & Zoning

The subject Real Estate is currently undeveloped.  The surrounding land uses are as follows:

- North: Estate residential lots zoned County R-2.5

- East: Undeveloped property zoned County R-1

- South: Undeveloped property zoned County R-2.5

- West: Undeveloped property zoned Regional Commercial and R-2

Infrastructure

All lots will be accessed through the internal public streets.  In accordance with the preliminary plan, this section will feature two entrances on CR 750N.  One stormwater detention facility will be constructed in accordance with the master drainage plan.  Sanitary sewer service will be provided by the Town, and water service will be provided by Citizen’s Energy Group.

Development Standards

The Jacobi Legacy Farms PUD includes its own Development Standards:

Area B

Setbacks	FY:  20’

SY:  5’

RY: 15’

Lot Width:  	53’

Lot Size: 	6,600 SF

Living Area: 	1,600/2,100 SF

Area C

Setbacks	FY:  25’

SY:  5’

RY: 15’

Lot Width:  	62’

Lot Size: 	7,700 SF

Living Area: 	1,800/2,400 SF

Perimeter Landscaping:  Perimeter landscaping and mounding is required along CR 750N.  The mounding is required to be a minimum of 3’ and up to 5’ and at least 7 trees and 6 shrubs shall be installed every 100 lineal feet.  The minimum width of this area shall be 30’, and at least 60% of the perimeter shall feature a farm style fence.  The petitioner’s plans meet this requirement.

Lot Landscaping: Each lot must include 3 trees, and 12 shrubs in the front yard. There are additional requirements for certain side yards.  This item is reviewed for after each home is constructed.

Architecture: All homes must comply with the Jacobi Legacy Farms PUD.  All homes will be reviewed on permit-by-permit basis when submitted for building permit review.

Lot Count: There is a maximum number of residential units within each Area of the PUD.  As future sections are proposed staff will be tracking lot counts to ensure the project does not exceed the maximum in each Area.

Street Lighting: Street lighting, consistent with the Town’s requirements, shall be provided.  The petitioner’s plans meet this requirement.

Pedestrian Accessibility:  The PUD requires internal sidewalks on both sides of all internal streets, internal paths in select locations, and a perimeter path along CR 750N.  The petitioner’s plans meet this requirement.

Open Space:  Open Space is required in accordance with the Preliminary Plan.  Open Space is tracked by staff on a section by section basis, thus far the subdivision is in compliance with the Open Space requirements.

Entrance Median: The PUD requires a median at each entrance, which are shown on the plans.  The petitioner’s plans meet this requirement.

Entrance Signage: The PUD requires primary signage monumentation at the entrance.  The petitioner’s plans show the primary monumentation.

Technical Advisory Committee (TAC)

The project went to TAC on March 3, 2022 and received a number of comments.  All comments applicable to the Plan Commission’s review have been addressed.  The petitioner will continue to work with staff on setting addresses, street names, and stormwater concerns.

Staff Comments:	Staff is supportive of this petition and recommends approval.  Following discussion, the Commission can motion to provide (a) approval, (b) continuance, or (c) denial.