Architectural Review Committee Staff Report				            June 21, 2022

Department of Planning and Building

Town of McCordsville

Project:		Crew Carwash

Petitioner: 		Crew Carwash

Request:	The petitioner is seeking approval of architecture for a carwash facility.

Staff Review:	This property is zoned Regional Commercial and is located in the Highway Overlay.  This report takes into account all architectural standards applicable to the building.   The list below denotes the applicable requirements and indicates whether or not the submittal meets the standard.

- Indicates compliance

- Indicates non-compliance

- Indicates further discussion needed

Italics indicate staff commentary.

- Façade colors shall be low reflectance, subtle, neutral hues or colors.  The use of high-fluorescent colors is prohibited.  Building trim and accent areas may feature brighter colors or primary colors.

- All buildings are to have a similar design or theme, materials, and compatible architecture.

- All siding must be stone, brick, cement fiber board, stucco, decorative pre-cast, architectural metal panels, matching approved materials in scale, integrally colored CMU, EIFS, or a combination of these materials.

- All buildings shall screen all mechanical equipment including that which is mounted to the roof and/or ground.  Mechanicals are not shown on the plans.  Staff requests the petitioner provide those locations and how they plan to screen the units.

- Any building elevation greater than 50’ in length shall have at least 1 projection or recess at least 3’ and extending at least 20% of the length of the elevation.  The ARC may approve wall plane recesses or projection designs that do not meet the above noticed minimums.

- Entryways at primary entrances shall include at least 3 of the following: (a) raised cornice parapets over the door, peaked roof forms, arches, or tile work, (b) integral planters or wing walls that incorporate landscaped areas and/or places for sitting, (c) enhanced exterior lighting, (d) prominent 3-D entryway feature, and/or (e) pilasters projecting from the plane of the wall.

- Trash receptacles enclosure.

- Commercial buildings of 15,000 SF or less shall incorporate 4-sided architecture.  All building elevations shall feature the same materials and level of architectural detailing as the front elevation.  Exceptions may be granted by the ARC for a side or rear elevation, which is not visible from an adjacent property, or an existing or planned private street or public right of way.

- All buildings shall use scale and design elements, such as but not limited to special lighting, awnings, trees, and other elements, to visually relate the building to the pedestrian scale and visually anchor the building to the ground/street level.

- For at least 60% of the building foundation perimeter, use any combination

of concrete, stone, or unique variation of color, texture, or material, at least 10” in height, around the base of the building.  Buildings may alternatively incorporate other architectural features such as ledges, façade revels, ground level fenestration, raised planters, or landscaping elements within 3’ of the finished grade.

- All front facades shall 3 of the following elements, all other facades shall have at least 1 element:

- Projection/recess

- Columns with trim or accent materials

- Change in finished material depths

- Change in material

- Building overhangs (as a feature, not standard overhang)

- Materials such as false windows or fenestration with architectural accents

- Other features as approved by the ARC

Staff Comments:	We noted our potential concern on mechanical units to Crew on Friday, June 17th.  We have apologized for the last-minute nature of that concern.  We are hopefully that we can work through that at the ARC meeting.  Additionally, staff would like to see some discussion between the ARC and the petitioner regarding the potential for the two overhead doors to be coated with a colored finish that is complimentary to the colors of the primary structure.