Architectural Review Committee Staff Report				    February 18, 2020

Department of Planning and Building

Town of McCordsville

Project:		Woodhaven

Petitioner: 		Fischer Homes

Request:	The petitioner is seeking approval of the addition of models/elevations to their current line-up in Woodhaven.

Staff Review:	The petitioner would like to introduce 3 new models and 20 new elevations (of existing models) into their Maple Street Series (R3 & R4 Areas of Woodhaven).

Maple Street Series

- Harper – American Classic, Modern Farmhouse, & Urban Modern

- Beacon – Cambridge Cottage, Modern Farmhouse, & Urban Modern

- Yosemite – American Classic, Modern Farmhouse, & Western Craftsman (brick)

- Cumberland – American Classic (brick), Coastal Classic (brick), & Urban Modern

- Greenbriar – Hyde Park Cottage (brick), Coastal Classic (brick), & Urban Modern

- Preston – Urban Modern, Western Craftsman, & Modern Farmhouse

- Denali – American Classic (brick), Urban Modern, & Modern Farmhouse

- Danville – Modern Farmhouse, Urban Modern, & Coastal Classic

- Wesley – Western Craftsman, Modern Farmhouse, & Urban Modern

*Bold denotes new model/elevation

Staff has conducted an architectural review of all of the submittal elevations based upon the Woodhaven Zoning Ordinance, and the commitment letter attached as Exhibit A of this report.  The list below denotes the basic requirements and indicates whether or not the submittal is in compliance:

- Indicates compliance

- Indicates non-compliance

- Indicates further discussion needed or a building permit review requirement

R3 & R4 Areas

  - All homes shall have a minimum 9” overhangs on all sides of the home

  - At least 50% of the homes shall have at least 50% brick on the front elevation (excluding windows, doors, garage doors, gables, and areas above first floor roofs)

  - Minimum roof pitch shall be 6:12

  - The min. size of ranch homes shall be 1,400 SF and min. size for two-story homes shall be 1,800 SF

  - Unless adjacent to masonry wrap, all windows, doors, and corners shall have a 1”x6” wood or vinyl surround, or shutters or decorative trim or decorative window header

Lot Specific Requirements:  To be reviewed for by staff on individual building permits, noted in this report for the petitioner’s reference:

  - Exterior fireplace chimneys shall be masonry unless placed on the rear façade

  - Half chimneys and direct vent gas fireplaces shall have a framed chase with a gable roof and vent on the side

  - Certain lots are required to have a minimum of 2 architectural features on the rear elevation, as denoted on Exhibit E of the Woodhaven zoning.

  - Homes on adjoining lots as well as one house most directly across the street shall not be constructed with the same front elevation or color.

  - A maximum of 2 ranch homes can be placed adjacent to each other.

  - No more than 10% of the homes shall have the same floor plan with the exact same front elevation.

Staff Comments:	In addition to the concerns noted in the above list, staff is also concerned with the Urban Modern elevations proposed by Fischer Homes.  While we really appreciate and support a builder, proposing something different and outside of the box we are concerned with introducing such a “different” product into a neighborhood that is more than 60% built-out.   While these elevations would certainly add to the diversity of housing in the neighborhood, our concern is based upon the introduction of this into a neighborhood of this “maturity”.  Staff does not feel the Urban Modern elevations are in keeping with the spirit and intent of the Woodhaven development.

Staff will present anti-monotony concerns at the meeting.