Architectural Review Committee Staff Report				      January 18, 2022

Department of Planning and Building

Town of McCordsville

Project:		Valvoline Instant Oil Change

Petitioner: 		Valvoline

Request:	The petitioner is seeking approval of architecture for a quick oil change facility.

Staff Review:	This property is zoned Regional Commercial and is located in the Highway Overlay.  This report takes into account all architectural standards applicable to the building.   The list below denotes the applicable requirements and indicates whether or not the submittal meets the standard.

- Indicates compliance

- Indicates non-compliance

- Indicates further discussion needed

Italics indicate staff commentary.

- Façade colors shall be low reflectance, subtle, neutral hues or colors.  The use of high-fluorescent colors is prohibited.  Building trim and accent areas may feature brighter colors or primary colors.

- All buildings are to have a similar design or theme, materials, and compatible architecture.

- All siding must be stone, brick, cement fiber board, stucco, decorative pre-cast, architectural metal panels, matching approved materials in scale, integrally colored CMU, EIFS, or a combination of these materials.

- All buildings shall screen all mechanical equipment including that which is mounted to the roof and/or ground.  Mechanicals are not shown on the plans.  Staff requests the petitioner provide those locations and how they plan to screen the units.

- Any building elevation greater than 50’ in length shall have at least 1 projection or recess at least 3’ and extending at least 20% of the length of the elevation.  The ARC may approve wall plane recesses or projection designs that do not meet the above noticed minimums.

- Sloped roofs shall not exceed an average height equal to that of 1.5 times the height of the supporting walls.

- Entryways at primary entrances shall include at least 3 of the following: (a) raised cornice parapets over the door, peaked roof forms, arches, or tile work, (b) integral planters or wing walls that incorporate landscaped areas and/or places for sitting, (c) enhanced exterior lighting, (d) prominent 3-D entryway feature, and/or (e) pilasters projecting from the plane of the wall.  The underlined element is present at the entrance and staff feels that for this type of facility the window wall present at the entrance should also be considered an element.  Staff would suggest enhanced exterior lighting as the third element.

- Trash receptacles enclosure.

- Commercial buildings of 15,000 SF or less shall incorporate 4-sided architecture.  All building elevations shall feature the same materials and level of architectural detailing as the front elevation.  Exceptions may be granted by the ARC for a side or rear elevation, which is not visible from an adjacent property, or an existing or planned private street or public right of way.

- Overhang eaves shall extend a minimum of 12” past the exterior of the exterior siding material. The overhang depth is not denoted on the plans.

- All buildings shall use scale and design elements, such as but not limited to special lighting, awnings, trees, and other elements, to visually relate the building to the pedestrian scale and visually anchor the building to the ground/street level.

- For at least 60% of the building foundation perimeter, use any combination

of concrete, stone, or unique variation of color, texture, or material, at least 10” in height, around the base of the building.  Buildings may alternatively incorporate other architectural features such as ledges, façade revels, ground level fenestration, raised planters, or landscaping elements within 3’ of the finished grade.

- All front facades shall 3 of the following elements, all other facades shall have at least 1 element:

- Projection/recess

- Columns with trim or accent materials

- Change in finished material depths

- Change in material

- Building overhangs (as a feature, not standard overhang)

- Materials such as false windows or fenestration with architectural accents

- Other features as approved by the ARC

Staff Comments:	The items denoted above are minor issues that can be addressed at the meeting.