Town Council Staff Report						                        June 14, 2022

Department of Planning and Building

Town of McCordsville

Project:		PC-22-006

Petitioner: 		Premier Land Company

Request:	The petitioner is seeking approval on a rezone from County IL to Rivendell PUD.

Staff Review:	The property is currently an agricultural field and is approximately 155 acres.  The assemblage has two frontages – CR 1000N and CR 400W.  This property also needs to be annexed into Town, and the Council held a first reading, public hearing, and adopted the fiscal plan in April.

Existing Land Use & Zoning

The subject parcel is approximately 155 acres and is undeveloped.  Adjacent properties are zoned as follows:

- North: Across CR 1000N, single family homes zoned SE Fishers PUD and undeveloped ag land zoned R-2.

- East: Across CR400W, undeveloped ag land zoned County IG

- South: Undeveloped land zoned County R-1 and County IL

- West: Undeveloped land zoned Liberty Ponds PUD

Infrastructure

The subject property is located within the Town’s sanitary sewer territory.    Water service is provided by Citizens Energy Group.  Vehicular site access will be provided via two (2) entrances, one on each frontage.  The plans also show interconnectivity into Haven Ponds and a street stub to the south.  The petitioner has proposed perimeter multi-use paths and internal paths and sidewalks. Drainage infrastructure will be installed in compliance with the Town’s requirements.

Development Proposal

The proposed subdivision proposes a maximum of 275 units on 177 acres for a maximum density of 1.77 units per acre.  The three closest subdivisions feature the following densities:

- Haven Ponds:		1.73 u/a

- Summerton		2.08 u/a

- McCord Pointe 		2.24 u/a

In keeping with many of the Town’s PUDs the petitioner is proposing a number of specific bulk standards that would apply within the PUD.  Highlights of those bulk standards are noted below:

Development Standards:

Area A

- Maximum Number of Lots			213

- Minimum Lot Area				12,000 Square Feet

- Minimum Lot Width			80 feet

- Minimum Front Yard Setback		30 feet (25’ on sec. frontage)

- Minimum Side Yard Setback		5 feet

- Minimum Rear Yard Setback		15 feet

- Minimum Square Footage			2,100/2,400 square feet

- Maximum Lot Coverage			45%

- Maximum Height-Principal			35 feet

- Minimum Building Separation		15 feet

Area B

- Maximum Number of Lots			62

- Minimum Lot Area				8,775 Square Feet

- Minimum Lot Width			65 feet

- Minimum Front Yard Setback		30 feet (25’ on sec. frontage)

- Minimum Side Yard Setback		6 feet

- Minimum Rear Yard Setback		15 feet

- Minimum Square Footage			1,700/2,200 square feet

- Maximum Lot Coverage			50%

- Maximum Height-Principal			35 feet

- Minimum Building Separation		12 feet

The petitioner’s proposal also includes a number of architectural, landscaping, and other design standards.  We will not list all those standards in this staff report, but they are denoted in the PUD Ordinance.

Staff Comments:	The subject property is located at the northeast extent of Town’s sanitary sewer territory.  It is divided by a legal drain, which acts as natural divider between the two product types.  One of the aspects of this site that was very important to staff was the creation of a grand entrance and streetscape.  The petitioner has agreed to provide this feature.  The petitioner’s design includes taking the existing legal drain along CR 1000N and turning it into a meandering creek feature that features boulders and landscaping.  Additionally, they have included extensive greenspace and prominent signage.  An exhibit illustrating the proposed entry signage is included in the petitioner’s proposal. The site design also includes a significant median entrance from 1000N that leads to the amenity center creating attractive entry design and viewpoint.

The project features approximately 49 acres of open space which is approximately 32% of the overall site.  This open space features a tree conservation area along the south property line, a legal drain, amenities, trails, open space, and detention.

At the time of this report, staff has not received any remonstrance letters.

For all rezones, Indiana Code Section 36-7-4-603 states that reasonable regard shall be paid to the following items:

- The Comprehensive Plan

- Current conditions and the character of the current structures and uses

- The most desirable use for which the land is adapted

- The conservation of property values throughout the jurisdiction

- Responsible growth and development

The Future Land Use Map envisions this assemblage to be developed as Low Density Residential.  According to the Comprehensive Plan Low Density Residential is intended for low density single-family residential uses.  The purpose of this classification is to create an attractive, stable, and orderly residential environment for citizens who desire larger lots. Development in this classification is often such that there are less than two dwelling units per acre.  The proposed project is in keeping with the Comprehensive Plan.

Staff is in support of this petition.